Smarty Weather Station

Final Project Documentation For ELET2415 Submission

Prepared date: April 21, 2026
ESP32 weather and soil monitoring system with Flask, MongoDB, and Vue dashboard

Prepared for ELET2415 project submission

Prepared date: April 21, 2026

Project folder: ELET2415/Project

## Document Control

| Item | Detail |
| --- | --- |
| Project name | Smarty Weather Station |
| System type | ESP32 IoT weather and soil monitoring station |
| Main hardware | ESP32 sensor board, ESP32-2432S028 touch gateway board, DHT22, BMP280, capacitive soil sensor |
| Main software | Arduino firmware, Flask backend, MongoDB, Vue/Vuetify frontend |
| Main communication path | Sensors -> ESP32 sensor node -> ESP-NOW -> ESP32 display/gateway -> Wi-Fi HTTP -> Flask -> MongoDB -> Vue dashboard |
| Primary source folders | hardware/, backend/, frontend/, presentation/assets/images/, Project Documentation/ |
| Submission document | This final consolidated documentation file |
| Git repository | https://github.com/RICK-MONK/Weather-Station |

## Table Of Contents

Executive Summary

Project Background And Objectives

Requirements And Scope

System Architecture

Bill Of Materials

Component List

Hardware Design

Schematics And Wiring Diagrams

Firmware Design

Backend Work

Frontend Work

Data Model And API Reference

Software And Hardware Diagrams

Setup And Operation

Testing And Validation

Results And Project Evidence

Troubleshooting, Risks, And Limitations

Future Improvements

Conclusion

Appendices

## 1. Executive Summary

Smarty Weather Station is an end-to-end IoT environmental monitoring project. It measures weather and soil conditions using ESP32-based hardware, sends readings wirelessly, stores data in a backend database, and displays live and historical readings through a web dashboard.

The project is more than a single-sensor serial monitor demonstration. It combines embedded firmware, sensor validation, ESP-NOW wireless transfer, local TFT display output, Flask API development, MongoDB storage, MQTT publishing support, and a Vue/Vuetify frontend. The final system gives the user both local visibility on the display node and remote visibility through a browser.

The system measures:

temperature

relative humidity

heat index

atmospheric pressure

estimated altitude derived from pressure

soil moisture as a calibrated relative percentage

The strongest engineering features are the system separation between sensor and gateway boards, validation of sensor readings, moving-average smoothing, last-valid-sample fallback behavior, backend timestamping, historical storage, dashboard trend views, and sensor-health flags.

## 2. Project Background And Objectives

Environmental monitoring is useful in agriculture, greenhouses, classrooms, laboratories, and smart-site monitoring. Many low-cost student sensor projects only display values locally or through the serial monitor. That makes the project useful for a proof of concept, but weak as a complete monitoring system.

This project addresses that gap by building a full sensing-to-dashboard pipeline. The goal is to collect data from multiple sensors, validate the data, move it wirelessly, store it centrally, and present it in a way that a user can understand.

### 2.1 Problem Statement

Environmental data is only useful if it can be trusted, accessed, and interpreted. A practical weather station must handle noisy readings, temporary sensor failure, wireless communication issues, backend storage, and user-facing visualization.

### 2.2 Project Objectives

Measure temperature and humidity with a DHT22 sensor.

Measure atmospheric pressure with a BMP280 sensor.

Estimate altitude from BMP280 pressure and a sea-level pressure reference.

Measure soil moisture using an analog soil moisture sensor.

Transfer readings between boards using ESP-NOW.

Forward readings to a Flask backend over Wi-Fi using HTTP.

Store readings in MongoDB with server-side timestamps.

Display live readings locally on a TFT display.

Display live, recent, and analyzed readings in a Vue dashboard.

Expose backend status and control actions through a frontend control page.

## 3. Requirements And Scope

### 3.1 Functional Requirements

| Requirement | Implementation |
| --- | --- |
| Collect temperature | DHT22 connected to ESP32 sensor node |
| Collect humidity | DHT22 connected to ESP32 sensor node |
| Calculate heat index | Firmware computes heat index from temperature and humidity |
| Collect pressure | BMP280 connected by I2C |
| Estimate altitude | Firmware derives altitude from pressure using SEA_LEVEL_PRESSURE_HPA |
| Collect soil moisture | Analog soil sensor connected to ADC GPIO |
| Transfer data locally | ESP-NOW packet from sensor node to display/gateway node |
| Display local readings | TFT display on gateway board |
| Forward to backend | HTTP POST to /api/weather/update |
| Store history | MongoDB collection configured by backend environment |
| Show dashboard | Vue/Vuetify frontend with dashboard, live, control, and analysis pages |
| Analyze data | Backend returns min, max, average, samples, and recent rows |

### 3.2 Non-Functional Requirements

The system should continue operating when one sensor temporarily fails.

The dashboard should report stale or delayed data rather than silently hiding it.

The backend should validate incoming data before storage.

Sensor data should be stored with consistent backend timestamps.

The frontend should support metric and imperial display modes.

### 3.3 In Scope

ESP32 firmware for sensing and display/gateway operation.

Weather payload validation and storage.

Live dashboard views.

Recent-history queries.

Date-range analysis queries.

Backend-tracked control actions.

Project schematics, wiring references, dashboard screenshots, and visual evidence from the project folder.

### 3.4 Out Of Scope For The Current Repository

User authentication.

Secure device provisioning.

Weatherproof enclosure.

Battery or solar power management.

Device-side execution of backend queued commands.

Formal automated regression test suite.

## 4. System Architecture

The project uses a layered IoT architecture:

sensing layer

local wireless transport layer

gateway and display layer

backend storage and API layer

browser dashboard layer

### 4.1 High-Level Architecture Diagram

Generated system architecture diagram

flowchart LR
    DHT[DHT22 temperature and humidity]
    BMP[BMP280 pressure]
    SOIL[Soil moisture sensor]
    SENSOR[ESP32 sensor node]
    ESPNOW[ESP-NOW wireless link]
    GATEWAY[ESP32 display and gateway]
    WIFI[Wi-Fi and HTTP]
    API[Flask API]
    DB[MongoDB]
    UI[Vue dashboard]

    DHT --> SENSOR
    BMP --> SENSOR
    SOIL --> SENSOR
    SENSOR --> ESPNOW
    ESPNOW --> GATEWAY
    GATEWAY --> WIFI
    WIFI --> API
    API --> DB
    API --> UI

Wireless weather station overview

### 4.2 End-To-End Data Flow

The sensor ESP32 reads the DHT22, BMP280, and soil sensor.

Firmware validates readings and rejects values outside expected ranges.

Firmware smooths accepted values using a moving average.

Firmware creates a SensorData payload.

The sensor node sends the payload to the gateway node through ESP-NOW.

The gateway displays readings on the TFT screen.

The gateway posts JSON to the Flask backend.

The Flask backend validates and normalizes the payload.

The backend stores the reading in MongoDB and attempts MQTT publishing.

The Vue frontend polls backend routes and updates dashboards, charts, and analysis views.

### 4.3 Communication Paths

| Source | Destination | Protocol | Purpose |
| --- | --- | --- | --- |
| Sensor node | Display/gateway node | ESP-NOW | Low-overhead local wireless data transfer |
| Gateway node | Flask backend | HTTP POST over Wi-Fi | Store latest reading |
| Frontend | Flask backend | HTTP GET/POST | Retrieve data and submit control actions |
| Backend | MQTT broker | MQTT publish | Optional secondary publish path after database save |

### 4.4 Active Operating Modes

| Mode | Description | Repository path |
| --- | --- | --- |
| PCB split system | Main submission architecture: sensor board sends ESP-NOW packets to display/gateway board | hardware/active/pcb_sensor_test/pcb_sensor_project/ and hardware/active/pcb_display_project/ |
| Generic ESP-NOW split system | General two-board sender/gateway sketches | hardware/active/espnow_weather/ |
| Standalone firmware path | Single-folder weather station firmware path | hardware/active/weather_station/ |
| Direct Wi-Fi sensor mode | Sensor can post directly to backend when configured | SENSOR_TRANSPORT_DIRECT_WIFI compile-time flag |

For the current PCB sensor sketch, SENSOR_TRANSPORT_DIRECT_WIFI is set to 0, so the active PCB path uses ESP-NOW to the display/gateway board.

The bill of materials below includes estimated hardware costs in USD based on representative online pricing checked on April 24, 2026. These figures are estimates for documentation purposes and exclude shipping and tax.

No purchase receipts or confirmed pricing data were found in the project folder. The bill of materials below therefore lists the confirmed hardware quantity, purpose, and interface based on the sketches, documentation, and schematic images in the repository.

| Item | Quantity | Estimated cost (USD) | Purpose | Interface / Notes |
| --- | --- | --- | --- | --- |
| ESP32 development board (sensor node) | 1 | 7.69 | Sensing and packet transmission | Wi-Fi, ESP-NOW, GPIO, ADC, I2C |
| ESP32-2432S028 touch display/gateway board | 1 | 13.99 | Local display, touch UI, and backend forwarding | Integrated ESP32 and touch TFT used by the gateway firmware |
| DHT22 temperature and humidity sensor | 1 | 8.90 | Measures ambient temperature and relative humidity | Digital signal on GPIO 4 |
| BMP280 pressure sensor module | 1 | 9.95 | Measures pressure and supports derived altitude estimate | I2C on GPIO 21 SDA and GPIO 22 SCL |
| Capacitive soil moisture sensor | 1 | 5.90 | Measures relative soil moisture | ADC input on GPIO 34 |
| USB power leads and small support parts | 1 set | 5.90 | Board power and basic hookup support | Includes USB leads used for programming and demonstration power |
| Wiring, headers, and connectors | 1 set | 3.95 | Sensor and board interconnection | Required for assembly and bench testing |
| Prototype breadboard hardware | 1 set | 4.95 | Temporary assembly and integration | Breadboard or equivalent prototyping hardware |
| Estimated hardware total | - | 61.23 | Estimated total before shipping and tax | Representative online prices checked April 24, 2026 |

## 6. Component List

### 6.1 ESP32 Sensor Node

Role:

reads the DHT22, BMP280, and soil sensor

validates and smooths readings

builds the SensorData payload

sends payload through ESP-NOW in the current PCB configuration

Key firmware file:

hardware/active/pcb_sensor_test/pcb_sensor_project/pcb_sensor_project.ino

### 6.2 ESP32 Display And Gateway Node

Role:

receives ESP-NOW packets

displays live values on the TFT

evaluates freshness and warning states

connects to Wi-Fi

posts weather JSON to Flask

Key firmware file:

hardware/active/pcb_display_project/pcb_display_project.ino

### 6.3 DHT22 Sensor

Measurements:

temperature in degrees Celsius

relative humidity in percent

Used for:

ambient temperature display

humidity display

computed heat index

### 6.4 BMP280 Sensor

Measurements:

atmospheric pressure in hPa

Derived value:

estimated altitude in meters

Important interpretation:

altitude is calculated from pressure and the configured sea-level reference

it should not be presented as a directly measured GPS-style altitude

### 6.5 Soil Moisture Sensor

Measurements:

raw ADC value

calibrated relative moisture percentage

Important interpretation:

the percentage is relative to the firmware calibration constants

it is not an absolute volumetric water-content measurement

### 6.6 TFT Display

Purpose:

local visualization without requiring a browser

displays weather, soil, system, and connection status screens in the gateway sketch

Current PCB note:

the PCB display sketch initializes touch-related constants but disables touch navigation with initializeTouchNavigation()

## 7. Hardware Design

### 7.1 Hardware Overview

The hardware design separates sensing from display and networking. The sensor node can be positioned near the measurement area, while the gateway node can be positioned where it can display data and reach the backend network.

ESP32 weather station device visual

### 7.2 Sensor PCB Pin Mapping

Generated hardware pinout diagram

| Signal | ESP32 GPIO | Interface | Purpose |
| --- | --- | --- | --- |
| DHT22 data | 4 | Digital | Temperature and humidity |
| BMP280 SDA | 21 | I2C data | Pressure sensor data |
| BMP280 SCL | 22 | I2C clock | Pressure sensor clock |
| Soil sensor analog output | 34 | ADC | Soil moisture raw value |

### 7.3 Display/Gateway Known Signals

| Signal | ESP32 GPIO / Setting | Purpose |
| --- | --- | --- |
| TFT backlight | GPIO 21 | Backlight enabled high in setup |
| ESP-NOW channel | 1 | Must align with sender channel |
| Backend port | 5000 | Flask API port |
| Project location label | Project Site | Display header metadata |
| NTP servers | pool.ntp.org, time.nist.gov | Gateway clock sync |

Additional TFT bus pin assignments are controlled by the local TFT_eSPI setup outside this repository.

### 7.4 Firmware Calibration Constants

| Constant | Value | Meaning |
| --- | --- | --- |
| SOIL_RAW_DRY | 720 | Dry-side soil calibration reference |
| SOIL_RAW_WET | 160 | Wet-side soil calibration reference |
| SOIL_SAMPLE_COUNT | 10 | Soil ADC samples averaged per read |
| SMOOTHING_WINDOW | 3 | Moving-average sample count |
| SEA_LEVEL_PRESSURE_HPA | 1013.25 | Pressure reference for estimated altitude |
| ESPNOW_WIFI_CHANNEL | 1 | ESP-NOW channel used by sensor and gateway |
| DHT_MAX_READ_RETRIES | 2 | DHT retry count |
| BMP280_MAX_READ_RETRIES | 3 | BMP retry count |

### 7.5 Sensor Range Validation

| Sensor / Field | Firmware validation |
| --- | --- |
| Temperature | -40 to 80 degrees Celsius |
| Humidity | 0 to 100 percent |
| Pressure | 300 to 1100 hPa |
| Estimated altitude | -150 to 4000 m |
| Soil raw ADC | 50 to 4000 |

## 8. Schematics And Wiring Diagrams

### 8.1 Project Schematic Reference

The schematic reference in the project folder shows an ESP32 weather station design with power regulation, indicator LEDs, programming header, sensor inputs, and ESP32 module connections.

Weather station schematic

Key schematic blocks:

power regulator from 5 V to 3.3 V

indicator LEDs

programming pins

humidity input

temperature and soil input

ESP32 module

### 8.2 Sensor Wiring Reference

The project includes wiring references for DHT22, BMP280, and soil sensor connections to an ESP32.

ESP32 DHT22 and BMP280 wiring

ESP32 soil and DHT wiring

### 8.3 Wireless Link Diagram

The wireless design separates the sensor node and the display/gateway node.

Wireless link concept

### 8.4 Hardware Block Diagram

+------------------+       ESP-NOW        +------------------------+
| ESP32 Sensor PCB | -------------------> | ESP32 Display Gateway  |
|                  |                      |                        |
| DHT22            |                      | TFT display            |
| BMP280           |                      | Wi-Fi client           |
| Soil sensor      |                      | HTTP POST client       |
+------------------+                      +-----------+------------+
                                                      |
                                                      | Wi-Fi HTTP
                                                      v
                                             +--------+---------+
                                             | Flask Backend    |
                                             | MongoDB Storage  |
                                             +--------+---------+
                                                      |
                                                      v
                                             +--------+---------+
                                             | Vue Dashboard    |
                                             +------------------+

## 9. Firmware Design

The firmware is divided across the sensor node and display/gateway node.

Generated firmware flow diagram

### 9.1 Firmware Source Files

| Firmware path | Purpose |
| --- | --- |
| hardware/active/pcb_sensor_test/pcb_sensor_project/pcb_sensor_project.ino | Current PCB sensor-node sketch |
| hardware/active/pcb_display_project/pcb_display_project.ino | Current PCB display/gateway sketch |
| hardware/active/espnow_weather/sensor_espnow_weather_sender/sensor_espnow_weather_sender.ino | Generic split sensor sender |
| hardware/active/espnow_weather/display_espnow_weather_gateway/display_espnow_weather_gateway.ino | Generic split display/gateway |
| hardware/active/weather_station/weather_station.ino | Standalone weather-station sketch |

### 9.2 Sensor Node Responsibilities

initialize DHT22, BMP280, soil ADC, Wi-Fi station mode, and ESP-NOW

read all sensors on a repeated loop

validate sensor values against safe ranges

retry DHT22 and BMP280 reads

average soil ADC samples

smooth accepted values using a 3-sample moving average

keep last-valid values when sensors fail temporarily

compute heat index

convert soil raw values to relative moisture percentage

send a SensorData struct every loop cycle

### 9.3 Sensor Startup Flow

Start serial logging
Initialize DHT22
Initialize I2C for BMP280
Initialize soil ADC
Try BMP280 address 0x76, then 0x77
Set Wi-Fi station mode
Set ESP-NOW channel
Print sender MAC and gateway MAC
Initialize ESP-NOW
Register gateway peer
Begin repeated sensing loop

### 9.4 SensorData Payload

| Field | Type | Meaning |
| --- | --- | --- |
| temperature | float | DHT22 temperature |
| humidity | float | DHT22 relative humidity |
| heatIndex | float | Calculated heat index |
| pressure | float | BMP280 pressure in hPa |
| altitude | float | Pressure-derived altitude estimate |
| seaLevelPressureHpa | float | Reference pressure used for altitude |
| soilMoisture | int | Raw soil ADC value inside the ESP-NOW struct |
| altitudeEstimated | uint8_t | Indicates estimated altitude field |
| dhtOk | uint8_t | DHT read health flag |
| bmpOk | uint8_t | BMP read health flag |
| soilOk | uint8_t | Soil read health flag |
| sampleMillis | uint32_t | Device millis() sample time |

Backend JSON also includes:

soilRaw

soilMoisturePercent

backend-generated timestamp

### 9.5 Gateway Firmware Responsibilities

initialize TFT display and backlight

connect to Wi-Fi

request NTP clock sync

initialize ESP-NOW receive mode

receive SensorData packets

merge partial incoming readings with previous readings

draw dashboard, soil, weather, and system screens

detect stale packets after 8000 ms

post JSON payloads to Flask approximately 4000 ms after packet receipt

### 9.6 Gateway Status Logic

The display/gateway classifies readings using these concepts:

| Status | Trigger example |
| --- | --- |
| NORMAL | All sensor classes are present and conditions are stable |
| WARNING | Dry soil, high humidity, or elevated condition |
| CRITICAL | Extreme temperature, dangerous heat index, or overflow risk |
| PARTIAL | One or more sensor classes are offline or fallback-based |

### 9.7 Firmware Reliability Features

DHT22 retry logic.

BMP280 retry and reinitialization logic.

Soil raw-range validation.

Moving-average smoothing.

Last-valid-sample fallback behavior.

Partial payload behavior instead of dropping all telemetry.

Serial logs for sensor health, payload content, Wi-Fi status, TCP tests, and HTTP response codes.

Sensor health flags forwarded to backend and frontend.

## 10. Backend Work

### 10.1 Backend Purpose

The backend is a Flask API service. It receives weather updates from the gateway or sensor, validates the payload, stores normalized documents in MongoDB, publishes saved readings through MQTT when configured, and exposes routes used by the frontend.

### 10.2 Backend Stack

| Package | Purpose |
| --- | --- |
| Flask | API server |
| Flask-CORS | Allows frontend API calls |
| PyMongo | MongoDB connection and queries |
| python-dotenv | Environment variable loading |
| paho-mqtt | MQTT publishing support |

### 10.3 Backend Source Files

| File | Role |
| --- | --- |
| backend/run.py | Starts the Flask app |
| backend/app/__init__.py | App factory and CORS setup |
| backend/app/views.py | Routes and payload validation |
| backend/app/functions.py | MongoDB access, normalization, and analysis |
| backend/app/control.py | In-memory control plane |
| backend/app/config.py | Environment-based configuration |
| backend/app/mqtt.py | MQTT connection and publish helper |

### 10.4 Weather Update Handling

Generated backend and frontend flow diagram

POST /api/weather/update performs this sequence:

Parse JSON body.

Check required fields id and type.

Confirm type equals weather.

Confirm optional sensor fields are numeric if present.

Normalize values into strings, floats, integers, and boolean flags.

Add backend UTC timestamp.

Insert document into MongoDB.

Attempt MQTT publish to the configured topic.

Return save result metadata to the device.

### 10.5 MongoDB Storage

Default backend database configuration:

| Variable | Default |
| --- | --- |
| MONGO_HOST | localhost |
| MONGO_PORT | 27017 |
| MONGO_DB | ELET2415 |
| MONGO_COLLECTION | weather |

Stored weather documents include the device ID, payload type, timestamp, present sensor values, and health flags.

### 10.6 Backend Analysis Work

The backend computes analysis for:

temperature

humidity

heat index

pressure

altitude

soil moisture percentage

The analysis route returns:

count of documents in range

start and end timestamps

min, max, and average per metric

sampled readings for charting

recent rows for tables

### 10.7 Backend Control Work

The backend control plane tracks:

maintenance mode

last alert acknowledgement

pending command queue

recent command history

delivery mode metadata

latest weather snapshot metadata

Supported control actions:

enable-maintenance

disable-maintenance

acknowledge-alerts

queue-gateway-refresh

queue-gateway-restart

queue-sensor-restart

clear-pending-commands

Important limitation:

control actions are backend-tracked only in this repository; the firmware does not currently pull and execute queued commands.

## 11. Frontend Work

### 11.1 Frontend Purpose

The frontend is a Vue/Vuetify browser dashboard that displays the current state of the weather station, recent trends, control status, and date-range analysis.

### 11.2 Frontend Stack

| Package | Purpose |
| --- | --- |
| Vue 3 | UI framework |
| Vue Router | Page routing |
| Pinia | Shared state store |
| Vuetify | UI components |
| Vite | Development server and build tool |
| @mdi/font | Icon font dependency |

### 11.3 Frontend Source Files

| File | Role |
| --- | --- |
| frontend/src/App.vue | App shell and navigation |
| frontend/src/router/index.js | Route map |
| frontend/src/store/weather.js | API calls, state, formatting, unit conversion |
| frontend/src/views/WeatherDashboard.vue | Main dashboard |
| frontend/src/views/WeatherLive.vue | Live sensor charts |
| frontend/src/views/WeatherControl.vue | Control and system status |
| frontend/src/views/WeatherAnalysis.vue | Date-range analysis |

### 11.4 Route Map

| Route | Purpose |
| --- | --- |
| /home | Main dashboard |
| /live/:sensor | Sensor-specific recent trend view |
| /control | Gateway/system operations view |
| /analysis/:sensor | Date-range analysis view |
| / | Redirects to /home |

### 11.5 Dashboard View

The dashboard displays:

latest reading freshness

last update time

reading age

temperature, humidity, heat index, pressure, altitude, and soil moisture cards

trend direction versus prior reading

sensor fallback status

recent history charts

Frontend dashboard screenshot

### 11.6 Live View

The live view presents recent-history chart pages for:

temperature

humidity

heat index

pressure

altitude

soil moisture

Frontend live screenshot

### 11.7 Control View

The control page displays:

gateway health

maintenance mode

queued commands

latest backend reading time

unit system toggle

pending commands

recent control activity

MongoDB configuration

schema metadata

frontend API log entries

Frontend control screenshot

### 11.8 Analysis View

The analysis page supports:

selectable start and end date range

metric-specific analysis

min, average, and max display

sampled chart readings

recent rows from the selected time range

Frontend analysis screenshot

### 11.9 Unit Conversion

The frontend supports metric and imperial rendering. Data is stored in the backend in the same normalized units, and display conversion happens in the frontend layer.

Conversions:

Celsius to Fahrenheit

hPa to inHg

meters to feet

## 12. Data Model And API Reference

### 12.1 Weather JSON Example

{
  "id": "620169874",
  "type": "weather",
  "temperature": 24.6,
  "humidity": 53.1,
  "heatIndex": 24.5,
  "pressure": 932.9,
  "altitude": 691.8,
  "seaLevelPressureHpa": 1013.25,
  "altitudeEstimated": 1,
  "soilMoisture": 372,
  "soilRaw": 372,
  "soilMoisturePercent": 62,
  "dhtOk": 1,
  "bmpOk": 1,
  "soilOk": 1,
  "sampleMillis": 182340
}

### 12.2 Stored Schema Fields

| Field | Type | Required | Source |
| --- | --- | --- | --- |
| id | string | Yes | Device |
| type | string | Yes | Device |
| temperature | float | No | DHT22 |
| humidity | float | No | DHT22 |
| heatIndex | float | No | Firmware calculation |
| pressure | float | No | BMP280 |
| altitude | float | No | Firmware estimate |
| soilMoisture | int | No | Device payload |
| seaLevelPressureHpa | float | No | Firmware constant |
| altitudeEstimated | bool | No | Firmware flag |
| soilRaw | int | No | Device payload |
| soilMoisturePercent | int | No | Firmware calculation |
| dhtOk | bool | No | Firmware health flag |
| bmpOk | bool | No | Firmware health flag |
| soilOk | bool | No | Firmware health flag |
| sampleMillis | int | No | Device runtime |
| timestamp | int | Yes | Backend |

### 12.3 API Routes

| Method | Path | Purpose |
| --- | --- | --- |
| GET | /api/health | Confirm backend is running |
| POST | /api/weather/update | Submit weather reading |
| GET | /api/weather/latest | Return latest stored reading |
| GET | /api/weather/recent?limit=50 | Return recent readings |
| GET | /api/weather/analysis | Return stats and sampled range data |
| GET | /api/control/status | Return control-plane status |
| POST | /api/control/action | Submit backend control action |
| GET | /api/system/info | Return database, schema, and route metadata |

## 13. Software And Hardware Diagrams

### 13.1 Software Flow Diagram

Generated end-to-end data sequence diagram

Sensor firmware
  -> read DHT22, BMP280, soil sensor
  -> validate ranges
  -> smooth values
  -> apply fallback if needed
  -> build SensorData
  -> send ESP-NOW packet

Gateway firmware
  -> receive SensorData
  -> update TFT display
  -> evaluate NORMAL/WARNING/CRITICAL/PARTIAL
  -> connect Wi-Fi
  -> POST JSON to Flask

Backend software
  -> validate JSON
  -> normalize data types
  -> add timestamp
  -> save to MongoDB
  -> publish to MQTT topic if available
  -> expose latest, recent, analysis, control, and system routes

Frontend software
  -> poll backend every 5 seconds
  -> normalize readings
  -> convert units for display
  -> render dashboard, live charts, control, and analysis pages

### 13.2 Backend Process Diagram

flowchart TD
    A[HTTP request /api/weather/update] --> B[Parse JSON]
    B --> C[Validate id and type]
    C --> D[Validate optional numeric fields]
    D --> E[Normalize payload]
    E --> F[Add backend timestamp]
    F --> G[Insert into MongoDB]
    G --> H[Attempt MQTT publish]
    H --> I[Return API response]

### 13.3 Sequence Diagram

sequenceDiagram
    participant S as Sensor Node
    participant G as Gateway Node
    participant B as Flask Backend
    participant M as MongoDB
    participant F as Frontend

    S->>S: Read and validate sensors
    S->>G: Send SensorData over ESP-NOW
    G->>G: Update TFT display
    G->>B: POST /api/weather/update
    B->>B: Validate and normalize payload
    B->>M: Insert weather document
    B-->>G: Return save response
    F->>B: Poll latest, recent, analysis, control
    B->>M: Query documents
    B-->>F: Return JSON data

### 13.4 Frontend Page Diagram

App.vue
  |
  +-- /home
  |     +-- WeatherDashboard.vue
  |
  +-- /live/:sensor
  |     +-- WeatherLive.vue
  |
  +-- /control
  |     +-- WeatherControl.vue
  |
  +-- /analysis/:sensor
        +-- WeatherAnalysis.vue

Shared data and API layer:
frontend/src/store/weather.js

## 14. Setup And Operation

### 14.1 Hardware Prerequisites

ESP32 sensor board

ESP32 display/gateway board

DHT22 sensor

BMP280 sensor

analog soil moisture sensor

TFT display

USB programming cables

stable power supply

### 14.2 Software Prerequisites

Python 3

Node.js and npm

MongoDB

Arduino IDE or arduino-cli

ESP32 board support package

Arduino libraries:

- DHT sensor library

- Adafruit BMP280 library

- TFT_eSPI

### 14.3 Backend Setup

From backend/:

python -m venv .venv
.venv\Scripts\activate
pip install -r requirements.txt
python run.py

The backend listens on 0.0.0.0:5000 by default.

### 14.4 Frontend Setup

From frontend/:

npm install
npm run dev

The Vite server listens on 0.0.0.0. By default, the frontend calls:

http://<current-browser-host>:5000

Set VITE_API_BASE_URL if the backend runs somewhere else.

### 14.5 Firmware Configuration Checklist

Before flashing:

confirm Wi-Fi SSID and password

confirm backend IP address constants

confirm device ID

confirm gateway MAC address for ESP-NOW mode

confirm ESPNOW_WIFI_CHANNEL is aligned

confirm TFT_eSPI board setup matches the display hardware

confirm soil calibration values

confirm BMP280 I2C address

### 14.6 Recommended Bring-Up Order

Start MongoDB.

Start the Flask backend.

Verify GET /api/health.

Start the Vue frontend.

Open the dashboard in a browser.

Flash and power the display/gateway board.

Flash and power the sensor board.

Watch serial logs on both boards.

Confirm backend inserts.

Confirm frontend cards and charts update.

## 15. Testing And Validation

The repository does not include a formal automated test suite. Validation is currently manual and system-level.

### 15.1 Backend Validation Checklist

Start Flask successfully.

Confirm GET /api/health.

Submit a known-good POST /api/weather/update.

Confirm MongoDB insert.

Confirm GET /api/weather/latest.

Confirm GET /api/weather/recent.

Confirm GET /api/weather/analysis.

Confirm GET /api/system/info.

Confirm each control action route.

### 15.2 Frontend Validation Checklist

Dashboard loads without console-breaking errors.

Latest reading cards populate.

Recent charts render.

Unit toggle changes displayed units.

Control page loads database and schema metadata.

Analysis page returns min, average, and max for a chosen date range.

### 15.3 Sensor Node Validation Checklist

DHT22 starts and returns in-range values.

BMP280 initializes at address 0x76 or 0x77.

Soil sensor raw value is inside the expected ADC range.

Smoothing produces stable output.

Fallback messages appear when a sensor is temporarily invalid.

Sender serial log shows payload activity every loop cycle.

### 15.4 Gateway Validation Checklist

Gateway receives ESP-NOW packets.

Packet age resets on new data.

TFT display updates correctly.

Stale status appears if packets stop arriving.

Backend POST succeeds after packet reception.

### 15.5 Integration Test Cases

| Test | Method | Expected result |
| --- | --- | --- |
| End-to-end happy path | Run sensor, gateway, backend, MongoDB, frontend | Reading appears on TFT, backend, and dashboard |
| Backend unavailable | Power boards while backend is offline | Local logs show HTTP failure; sensing/display continues |
| DHT failure | Disconnect or destabilize DHT22 | dhtOk false; fallback used if baseline exists |
| BMP failure | Disconnect BMP280/I2C | bmpOk false; other sensors continue |
| Soil instability | Disturb or disconnect soil probe | out-of-range protection and fallback behavior |
| Stale ESP-NOW data | Stop sensor sender | gateway marks packet stale after threshold |

## 16. Results And Project Evidence

### 16.1 Local Display Evidence

The project folder includes a TFT/display prototype reference image.

Prototype display photo

### 16.2 Dashboard Evidence

The frontend screenshots show the dashboard receiving current readings, reporting reading age, and displaying recent trend graphs.

Frontend dashboard screenshot

### 16.3 Control Page Evidence

The control page demonstrates backend status, queued commands, database metadata, and system operations.

Frontend control screenshot

### 16.4 Analysis Evidence

The analysis page demonstrates date-range statistics and sampled readings.

Frontend analysis screenshot

### 16.5 Component Evidence

[Image could not be inserted: ../presentation/assets/images/esp32-board.jpg; ]

[Image could not be inserted: ../presentation/assets/images/dht22.jpg; ]

BMP280 sensor

Soil sensor

TFT display

## 17. Troubleshooting, Risks, And Limitations

### 17.1 Common Troubleshooting

| Problem | Likely causes | Checks |
| --- | --- | --- |
| Backend does not receive data | Wrong backend IP, backend offline, Wi-Fi issue, port blocked | Confirm Flask, /api/health, firmware IP, serial logs |
| ESP-NOW packets not arriving | Channel mismatch, wrong peer MAC, Wi-Fi channel conflict | Confirm ESPNOW_WIFI_CHANNEL, MAC addresses, serial logs |
| BMP280 fails | Wiring, I2C address, power issue | Check SDA/SCL, addresses 0x76 and 0x77, startup logs |
| DHT22 read failures | Wiring instability, timing sensitivity | Check data line, supply voltage, retry logs |
| Soil readings wrong | Calibration mismatch, ADC range issue, probe placement | Inspect raw ADC, recalibrate wet/dry constants |
| TFT display incorrect | TFT_eSPI setup mismatch | Confirm local TFT driver and pin configuration |
| Frontend stale or empty | No backend data, wrong API base URL, backend query failed | Check API logs and /api/weather/latest |
| Control actions do not affect devices | Device-side command consumer not implemented | Treat as known current limitation |

### 17.2 Known Risks

Firmware network settings are hard-coded and must be updated when the Wi-Fi or backend host changes.

ESP-NOW and Wi-Fi coexistence depends on channel alignment.

Control state is stored in backend memory and is lost on backend restart.

The current API has no authentication.

Soil moisture is relative and probe-dependent.

Altitude is estimated from pressure and calibration.

There is no persistent local queue if backend posting fails.

### 17.3 Current Limitations

No automated frontend/backend/firmware test suite in the repository.

No device-side command pickup for queued backend actions.

No weatherproof enclosure or power-management design.

No long-duration endurance test results included.

TFT bus configuration depends on local TFT_eSPI setup outside the repository.

## 18. Future Improvements

Add a weatherproof enclosure.

Add battery or solar power support.

Persist control state in MongoDB.

Add device-side polling/execution for queued commands.

Add backend authentication or LAN access control.

Add alerting through email, SMS, or push notification.

Add an automated irrigation actuator.

Add configuration mode for Wi-Fi and backend address.

Add automated tests for API routes and frontend store behavior.

Add long-duration logging and reliability testing.

## 19. Conclusion

Smarty Weather Station demonstrates a complete IoT monitoring system. It reads multiple environmental sensors, validates and smooths measurements, sends data over ESP-NOW, displays readings locally, forwards data to a Flask backend, stores readings in MongoDB, and visualizes live and historical values in a Vue dashboard.

The project shows integration across hardware, firmware, backend, database, and frontend layers. It also addresses practical engineering concerns such as noisy sensors, partial failures, stale data, backend validation, and user-facing status reporting. The result is a strong project platform that can be extended into smart agriculture, greenhouse monitoring, classroom demonstration, and environmental automation.

## 20. Appendices

### Appendix A: Main Repository Files Used

| Path | Use In This Document |
| --- | --- |
| README.md | Repository overview |
| Project Documentation/01-project-overview.md | Project goals and scope |
| Project Documentation/02-system-architecture.md | Architecture and data lifecycle |
| Project Documentation/03-hardware-design.md | Components, pin mapping, calibration |
| Project Documentation/04-firmware-design.md | Firmware structure and behavior |
| Project Documentation/05-backend-design.md | Backend architecture |
| Project Documentation/06-frontend-design.md | Frontend architecture |
| Project Documentation/07-api-reference.md | API routes and schema |
| Project Documentation/08-setup-and-operation.md | Setup steps |
| Project Documentation/09-testing-and-validation.md | Validation strategy |
| Project Documentation/10-troubleshooting-and-risks.md | Troubleshooting and risks |
| docs/weather-station-report.md | Report draft source |
| hardware/active/pcb_sensor_test/pcb_sensor_project/pcb_sensor_project.ino | Current sensor firmware constants and behavior |
| hardware/active/pcb_display_project/pcb_display_project.ino | Current gateway/display firmware constants and behavior |
| backend/app/views.py | API routes and validation logic |
| backend/app/functions.py | MongoDB normalization and analysis |
| frontend/src/store/weather.js | Frontend API, unit conversion, and normalization |
| frontend/src/views/ | Dashboard, live, control, and analysis UI |

### Appendix B: Image And Diagram Assets Used

| Asset | Purpose |
| --- | --- |
| presentation/assets/images/wireless-weather-station.png | System overview visual |
| presentation/assets/images/weather-station-device.png | Display/device visual |
| presentation/assets/images/wireless-link.png | Wireless display/gateway concept |
| presentation/assets/images/weather-station-schematic.jpg | Schematic reference |
| presentation/assets/images/esp32-dht-bmp-wiring.jpg | DHT22 and BMP280 wiring reference |
| presentation/assets/images/esp32-soil-dht-wiring.jpg | Soil and DHT wiring reference |
| presentation/assets/images/frontend-home.png | Dashboard screenshot |
| presentation/assets/images/frontend-live.png | Live view screenshot |
| presentation/assets/images/frontend-control.png | Control page screenshot |
| presentation/assets/images/frontend-analysis.png | Analysis page screenshot |
| presentation/assets/images/prototype-display-photo.jpg | TFT/display prototype photo |
| presentation/assets/images/esp32-board.jpg | ESP32 component image |
| presentation/assets/images/dht22.jpg | DHT22 component image |
| presentation/assets/images/bmp280.jpg | BMP280 component image |
| presentation/assets/images/soil-sensor.jpg | Soil sensor component image |
| presentation/assets/images/tft-display.jpg | TFT display component image |
| Project Documentation/final-documentation-assets/system-architecture-generated.png | Generated final architecture diagram for submission |
| Project Documentation/final-documentation-assets/hardware-pinout-generated.png | Generated hardware pinout and interface diagram |
| Project Documentation/final-documentation-assets/firmware-flow-generated.png | Generated firmware flow diagram |
| Project Documentation/final-documentation-assets/backend-frontend-flow-generated.png | Generated backend/frontend flow diagram |
| Project Documentation/final-documentation-assets/data-sequence-generated.png | Generated end-to-end sequence diagram |

The project folder also contains SVG diagram assets:

presentation/assets/images/system-architecture.svg

presentation/assets/images/wiring-diagram.svg

presentation/assets/images/schematic-overview.svg

presentation/assets/images/espnow-flow.svg

presentation/assets/images/backend-flow.svg

presentation/assets/images/data-flow-animated.svg

### Appendix C: Backend Environment Variables

FLASK_HOST=0.0.0.0
FLASK_PORT=5000
FLASK_DEBUG=true
MONGO_HOST=localhost
MONGO_PORT=27017
MONGO_DB=ELET2415
MONGO_COLLECTION=weather
MQTT_BROKER=localhost
MQTT_PORT=1883
MQTT_TOPIC=weatherstation/update

### Appendix D: Submission Notes

This document is based only on project-folder resources and current source code.

BOM costs are estimated from representative online component listings checked on April 24, 2026, and they exclude shipping and tax.

The document describes the PCB split system as the main submission architecture because it matches the active PCB sensor and display sketches.

Some image assets are project visuals, screenshots, references, or presentation graphics. Actual assessment photos can be added later if more project-build photos are captured.